Digital 800/900/1800/2100/2600MHz  Band-adjustable Repeater

Model: AT-012-LGDWF-20

## Product Features:

- Digital bandwidth-adjustable features allow for seamless adaptability to different frequency bands.

- LCD display the input/output signal strength.

- Auto isolation detection.

- Improve any three mobile networks at the same time.

- Built-in Auto Gain Control (AGC)& Auto Level Control (ALC).

- Center frequency movable.

- OMS Lite for remote/local control and monitoring(Optional)

## Working Schematic Diagram

## Application Scenario:

## Technical Specifications:

| P/N: AT-012-LGDWF-20 |  | B20 | B8 | B3 | B1 | B7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Range (MHz,adjustable) | Uplink | 832-862 | 880-915 | 1710-1785 | 1920-1980 | 2500-2570 |  |
|  | Downlink | 791-821 | 925-960 | 1805-1880 | 2110-2170 | 2620-2690 |  |
| Number of Sub-bands |  | 1 | 1 | 1 | 1 | 1 |  |
| Bandwidthper Sub-band(MHz,adjustable) |  | 0.2-20 | 0.2-20 | 0.2-20 | 0.2-20 | 0.2-20 |  |
| Max.Gain(dB) | Uplink | 65±2 dB | 65±2 dB | 65±2 dB | 65±2 dB | 65±2 dB |  |
|  | Downlink | 70±2 dB | 70±2 dB | 70±2 dB | 70±2 dB | 70±2 dB |  |
| Manual Gain Control |  | 31 dB in step of 1dB |  |  |  |  |  |
| Automatic Gain Control |  | ≥20 dB |  |  |  |  |  |
| Gain Flatness(per sub-band) |  | ≤±2.5 dB(peak-to-peak) |  |  |  |  |  |
| Max.Input Power Without Damage |  | 0 dBm |  |  |  |  |  |
| Output Power(dBm) | Uplink | 10±2 | 10±2 | 10±2 | 10±2 | 10±2 | 10±2 |
|  | Downlink | 20±2 | 20±2 | 20±2 | 20±2 | 20±2 | 20±2 |
| Out of Band Gain |  | 2.5≤f_offset_CW<5.0 MHz ≤60 dB |  |  |  |  |  |
|  |  | 5.0≤f_offset_CW<10.0 MHz ≤45 dB |  |  |  |  |  |
|  |  | 10.0 MHz≤f_offset_CW ≤35 dB |  |  |  |  |  |
| Spurious Emission | 9KHz-1GHz | ≤-36dBm |  |  |  |  |  |
|  | 1GHz-12.75GHz | ≤-30dBm |  |  |  |  |  |
| ACRR | ±10/20MHz | ≤-36dBc/30KHz |  |  |  |  |  |
|  | ±20/40MHz | ≤-40dBc/30KHz |  |  |  |  |  |
| EVM |  | ≤8% | ≤8% | ≤8% | ≤8% | ≤8% | ≤8% |
| Frequency Stability |  | ≤±0.01 ppm |  |  |  |  |  |
| Noise Figure |  | ≤6dB |  |  |  |  |  |
| VSWR |  | ≤1.8 |  |  |  |  |  |
| System Delay |  | ≤6μs |  |  |  |  |  |
| RF Connector |  | N-Female |  |  |  |  |  |
| Impedance |  | 50Ω |  |  |  |  |  |
| Power Supply |  | Input:AC100~240V,50/60 Hz;Output:DC24V/4.2A |  |  |  |  |  |
| Power Consumption |  | ≤130W |  |  |  |  |  |
| Dimensions |  | 350*268*58mm |  |  |  |  |  |
| Weight |  | ≤12kgs |  |  |  |  |  |
| IP Rating |  | IP30 |  |  |  |  |  |
| Operating Temperature |  | -10℃ to60℃ |  |  |  |  |  |
| NMS | Local | Via Type-C/Wi-Fi App |  |  |  |  |  |
|  | Remote | Via Cloud-based NMS(optional) |  |  |  |  |  |

Technical specification is subject to change without prior notice.