Digital Tri Band 1800+2100+3500MHz Pico Band-adjustable Repeater

Model: AT-DS-DWN78-20

## Product Features:

- Digital bandwidth-adjustable features allow for seamless adaptability to different frequency bands.

- LCD display the input/output signal strength.

- Auto isolation detection.

- Improve any three mobile networks at the same time.

- Built-in Auto Gain Control (AGC)& Auto Level Control (ALC).

- Center frequency movable.

- OMS Lite for remote/local control and monitoring(Optional)

## Working Schematic Diagram

## Application Scenario:

## Technical Specifications:

| AT-DS-DWN78-20; B3(1800); B1(2100); N78(3500). |
| --- |
| Frequency Range (MHz,adjustable) | Uplink (adjustable) | 1710-1730 MHZ | 1920-1940MHZ | 3450-3550 MHZ |
|  |  | 1730-1750 MHZ | 1940-1960MHZ |  |
|  |  | 1750-1780 MHZ | 1960-1980MHZ |  |
|  | Downlink (adjustable) | 1805-1825 MHZ | 2110-2130MHZ | 3450-3550 MHZ |
|  |  | 1825-1845 MHZ | 2130-2150MHZ |  |
|  |  | 1845-1875 MHZ | 2150-2170MHZ |  |
| Number of Sub-bands |  | 3 | 3 |  |
| Bandwidth per sub-band |  | 5-75 MHz adjustable | 5-60MHz adjustable | 5-100 MHz adjustable |
| Max.Gain(dB) | Uplink | 70士2dB | 70士2dB | 70士2dB |
|  | Downlink | 75士2dB | 75士2dB | 75士2dB |
| Manual Gain Control |  | 31 dB in step of 1dB |  |  |
| Automatic Gain Control |  | ≥20 dB |  |  |
| Gain Flatness(per sub-band) |  | ≤±3.5 dB(peak-to-peak) |  |  |
| Max.Input Power Without Damage |  | 0 dBm |  |  |
| Output Power(dBm) | Uplink | 20±2dBm | 20±2dBm | 20±2dBm |
|  | Downlink | 23±2dBm | 23±2dBm | 23±2dBm |
| out of Band Gain | 0.2≤f offset cw<1.0MHz | ≤60dB |  |  |
|  | 1.0≤f offset cw<5.0MHz | ≤45dB |  |  |
|  | 5.0≤f offset cw<10.0MHz | ≤45dB |  |  |
|  | 10MHz≤f_offset_cw | ≤35dB |  |  |
| Spurious Emission | 9KHz-1GHz | ≤-36dBm |  |  |
|  | 1GHz-12.75GHz | ≤-30dBm |  |  |
| ACRR | ±10/20MHz | ≤-36dBc/30KHz |  |  |
|  | ±20/40MHz | ≤-40dBc/30KHz |  |  |
| EVM |  | ≤6% | ≤6% | ≤3.5% |
| Frequency Stability |  | ≤±0.01 ppm |  |  |
| Noise Figure |  | ≤8dB |  |  |
| VSWR |  | ≤2 |  |  |
| System Delay |  | ≤6μs |  |  |
| RF Connector |  | N-Female |  |  |
| Impedance |  | 50Ω |  |  |
| Power Supply | Input | AC100~240V,50/60 Hz |  |  |
|  | Output | DC24V/4.2A |  |  |
| Power Consumption |  | ≤70W |  |  |
| Dimensions |  | 330*258*95mm |  |  |
| Weight |  | ≤7kgs |  |  |
| IP Rating |  | IP30 |  |  |
| Operating Temperature |  | -10℃ to 60℃ |  |  |
| NMS | Local | RJ45 |  |  |
|  | Remote | Via Cloud-based NMS(optional) |  |  |

Technical specification is subject to change without prior notice.