25dBm  900+1800+2100+2600Hz Repeater

Model: AT-25-GDWL26

## Product Features:

- LCD display the input/output signal strength.

- Auto isolation detection.

- Improve any three mobile networks at the same time.

- Built-in Auto Gain Control (AGC)& Auto Level Control (ALC).

- OMS Lite for remote/local control and monitoring(Optional)

## Working Schematic Diagram

## Application Scenario:

## Technical Specifications:

| P/N: AT-25-GDWL26 |  | B7 (2600MHz) |  | B8 (900MHz) | B3 (1800MHz) | B1(2100MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range | Uplink | 2500-2570MHz |  | 880-915MHz | 1710-1785MHz | 1920-1980MHz |
|  | Downlink | 2620-2690MHz |  | 925-960MHz | 1805-1880MHz | 2110-2170MHz |
| Max. Gain | Uplink | 95±3 dB | 95±3 dB | 95±3 dB | 95±3 dB | 95±3 dB |
|  | Downlink | 97±3 dB | 97±3 dB | 97±3 dB | 97±3 dB | 97±3 dB |
| Manual Gain Control |  | 31 dB in step of 1 dB |  |  |  |  |
| Automatic Gain Control |  | ≧ 20 dB |  |  |  |  |
| Gain Flatness (per sub-band) |  | ≦ ±3 dB (peak-to-peak) |  |  |  |  |
| Max. Input Power Without Damage |  | 0 dBm |  |  |  |  |
| Output Power | Uplink | 22±2 dBm |  |  |  |  |
|  | Downlink | 25±2 dBm |  |  |  |  |
| Spurious Emission | 9KHz-1GHz | ≦ -36dBm |  |  |  |  |
|  | 1GHz-12.75GHz | ≦ -30dBm |  |  |  |  |
| ACRR | ±20/100MHz | ≦-36dBc/30KHz |  |  |  |  |
|  | ±40/200MHz | ≦-40dBc/30KHz |  |  |  |  |
| EVM |  | ≦ 8% |  |  |  |  |
| Frequency Stability |  | ≦ ±0.01 ppm |  |  |  |  |
| Noise Figure |  | ≦ 6 dB |  |  |  |  |
| VSWR |  | ≦ 2 |  |  |  |  |
| System Delay |  | ≦ 4.5 µs |  |  |  |  |
| RF Connector |  | N-Female |  |  |  |  |
| Impedance |  | 50 Ω |  |  |  |  |
| Power Supply |  | DC24V5A |  |  |  |  |
| Power Consumption |  | ≦ 200W |  |  |  |  |
| Dimensions |  | 480*350*88mm |  |  |  |  |
| Weight |  | ≦ 12kgs |  |  |  |  |
| IP Rating |  | IP40 |  |  |  |  |
| Operating Temperature |  | -25 °C to 55 °C |  |  |  |  |
| Control & Monitoring | Remote | LTE Modem |  |  |  |  |

Technical specification is subject to change without prior notice.