27dBm Dual Band 1800+2100MHz Repeater

Model: AT-1401-DW

## Product Features:

- Improve any two mobile networks at the same time

- LCD display the input/output signal strength

- Auto isolation detect function

- Auto gain control ,auto level control

- Auto uplink noise reduction,avoid disturbing to BTS

- Smart LCD to guide the installation

- Metal industrial design

## Working Schematic Diagram

## Application Scenario:

## Technical Specifications:

| P/N: AT-1401-DW |  | B3 (1800MHz) | B1(2100MHz) |
| --- | --- | --- | --- |
| Frequency Range | Uplink | 1710 - 1785 MHz | 1920-1980 MHz |
|  | Downlink | 1805 - 1880 MHz | 2110-2170 MHz |
| Max. Gain | Uplink | 70±3 dB | 70±3 dB |
|  | Downlink | 75±3 dB | 75±3 dB |
| Manual Gain Control |  | 31 dB in step of 1 dB |  |
| Automatic Gain Control |  | ≧ 20 dB |  |
| Max. Input Power Without Damage |  | 0 dBm |  |
| Output Power | Uplink | 23±2 dBm | 23±2 dBm |
|  | Downlink | 27±2 dBm | 27±2 dBm |
| Spurious Emission | 9KHz-1GHz | ≦ -36dBm |  |
|  | 1GHz-12.75GHz | ≦ -30dBm |  |
| ACRR | ±20/100MHz | ≦-36dBc/30KHz | ≦-36dBc/30KHz |
|  | ±40/200MHz | ≦-40dBc/30KHz | ≦-40dBc/30KHz |
| EVM |  | ≦ 8% | ≦ 8% |
| Frequency Stability |  | ≦ ±0.01 ppm | ≦ ±0.01 ppm |
| Noise Figure |  | ≦ 9 dB |  |
| VSWR |  | ≦ 2 |  |
| System Delay |  | ≦ 1 .6µs |  |
| RF Connector |  | N-Female |  |
| Impedance |  | 50 Ω |  |
| Power Supply |  | DC 24V |  |
| Power Consumption |  | ≦ 120 W |  |
| Dimensions |  | 267*235*52cm |  |
| Weight |  | ≦ 3 kg |  |
| IP Rating |  | IP43 |  |
| Operating Temperature |  | -25 °C to 80 °C |  |

Technical specification is subject to change without prior notice.