Band 28 700MHz Wid band Repeater

Model: AS-L

## Product Features:

- Improve any two mobile networks at the same time

- LCD display the input/output signal strength

- Auto isolation detect function

- Auto gain control ,auto level control

- Auto uplink noise reduction,avoid disturbing to BTS

- Smart LCD to guide the installation

## Working Schematic Diagram

Application Scenario:

## Technical Specifications:

| P/N: AS-L |  | B28 ( 700MHz) |
| --- | --- | --- |
| Frequency Range | Uplink | 703-748MHz |
|  | Downlink | 758-803MHz |
| Bandwidth |  | 35MHz |
| Max. Gain | Uplink | 60±3 dB |
|  | Downlink | 70±3 dB |
| Manual Gain Control |  | 31 dB in step of 1 dB |
| Automatic Gain Control |  | ≧ 20 dB |
| Gain Flatness (per sub-band) |  | ≦ ±3 dB (peak-to-peak) |
| Max. Input Power Without Damage |  | 0 dBm |
| Output Power | Uplink | 17±2 dBm |
|  | Downlink | 20±2 dBm |
| Spurious Emission | 9KHz-1GHz | ≦ -36dBm |
|  | 1GHz-12.75GHz | ≦ -30dBm |
| ACRR | ±20/100MHz | ≦-36dBc/30KHz |
|  | ±40/200MHz | ≦-40dBc/30KHz |
| EVM |  | ≦ 8 % |
| Frequency Stability |  | ≦ ±0.01 ppm |
| Noise Figure |  | ≦ 8 dB |
| VSWR |  | ≦ 2 |
| System Delay |  | ≦ 3 µs |
| RF Connector |  | SMA-Female |
| Impedance |  | 50 Ω |
| Power Supply |  | DC 5V2A |
| Power Consumption |  | ≦ 15 W |
| Dimensions |  | 165*115*35mm |
| Weight |  | ≦ 0.5 kgs |
| IP Rating |  | IP40 |
| Operating Temperature |  | -25 °C to 55 °C |

Technical specification is subject to change without prior notice.