UHF380-470MHz Active DAS System

Model: AT-OPT-UHF380

## Product Features:

- Support multi-band system.

- Optical transmission up to 10km (max.).

- Support OMS Lite local/remote control.

- Low noise and high reliability.

- Each master unit can support max.16 remote units.

- Support to monitor donor signal parameters for easy optimization and troubleshooting.

## Application Scenario:

## Technical Specifications:

| Specifications |  |
| --- | --- |
| Part Number | AT-OPT-UHF380MU-AIR |
| Duplex Frequency Range | 380-470MHz |
| Operational Bandwidth | 0.2-5MHz |
| Maximum Gain (UL/DL) | 40/40+3 dB |
| Pass Band Ripple | < +2dB |
| Optical Output Power | -6dBm+2dB @1550nm |
| Optical Receiver Sensitivity | > -15dBm |
| Uplink Output Power | 23dBm + 2dB |
| Manual Gain Control | 0 - 31dB @1dB step |
| Auto Level Control | > 20dB |
| Group Delay Time | < 1.5μs |
| VSWR | < 1.5 |
| Noise Figure | < 6dB (uplink, with 1RU) |
| Spurious Emissions | Comply with 3GPP TS36.106 |
| IMD3 | > 45dBc |
| Connector | 1*N-female, 8*LC/UPC duplex |
| Power Consumption | < 40W |
| Power Supply | AC100-240V, comply with IEC62368-1 |
| Local Control | LCD+Keypad, WiFi, RJ45 |
| OMC (Optional) | LTE Modem |
| Approx. Weigh | 10kg |
| Operating Temperature | -10 to + 55°C |
| Applications | Indoor (IP40) |
| Humidity | 5% to 95% |
| MTBF | > 50,000 hours |
| Installation | Rack mount |
| Dimension | Standard 2U (19” rack) |

| Specifications |  |
| --- | --- |
| Part Number | AT-OPT-UHF380RU-AIR |
| Duplex Frequency Range | 380-470MHz |
| Operational Bandwidth | 0.2-5MHz |
| Maximum Gain (UL/DL) | 45/40+3 dB |
| Auto Gain Control Range | 31dB |
| Pass Band Ripple | < +2dB |
| Optical Output Power | 3dBm+2dB @1310nm |
| Optical Receiver Sensitivity | > -15dBm |
| Uplink Output Power | 33dBm + 2dB |
| Auto Gain Control | 0 - 31dB @1dB step |
| Auto Level Control | > 20dB |
| Group Delay Time | < 1μs |
| VSWR | <1.5 |
| Spurious Emissions | Comply with 3GPP TS36.106 |
| IMD3 (2X37dBm) | < 45dBc |
| Connector | 1*N-female, 1*LC/UPC |
| Power Consumption | < 50W |
| Power Supply | AC100-240V, comply with IEC62368-1 |
| Local Control | LCD+ Keypad, WiFi or RJ45 |
| OMC (Optional) | Through MU via fiber |
| Approx. Weigh | 10kg |
| Operating Temperature | -10 to + 55°C |
| Applications | Indoor& outdoor (IP65) |
| Humidity | 5% to 95% |
| MTBF | > 50,000 hours |
| Installation | Wall mount |
| Dimension (without connector) | 385mm*307mm*204mm |

Technical specification is subject to change without prior notice.