900+1800MHz Wid band Repeater

Model: AT-900+1800

## Product Features:

- Improve any two mobile networks at the same time

- LCD display the input/output signal strength

- Auto isolation detect function

- Auto gain control ,auto level control

- Auto uplink noise reduction,avoid disturbing to BTS

- Smart LCD to guide the installation

- Metal industrial design

## Working Schematic Diagram

Application Scenario:

## Technical Specifications:

| P/N: AT-1800+3500 |  | B3 (1800MHz) | B8 (900MHz) |
| --- | --- | --- | --- |
| Frequency Range | Uplink | 1710-1785MHz | 880-915MHz |
|  | Downlink | 1805-1880MHz | 925-960MHz |
| Bandwidth |  | 75MHz | 35MHz |
| Max. Gain | Uplink | 60±3 dB | 60±3 dB |
|  | Downlink | 70±3 dB | 70±3 dB |
| Manual Gain Control |  | 31 dB in step of 1 dB |  |
| Automatic Gain Control |  | ≧ 20 dB |  |
| Gain Flatness (per sub-band) |  | ≦ ±3 dB (peak-to-peak) |  |
| Max. Input Power Without Damage |  | 0 dBm |  |
| Output Power | Uplink | 15±2 dBm | 15±2 dBm |
|  | Downlink | 17±2 dBm | 18±2 dBm |
| Spurious Emission | 9KHz-1GHz | ≦ -36dBm |  |
|  | 1GHz-12.75GHz | ≦ -30dBm |  |
| ACRR | ±20/100MHz | ≦-36dBc/30KHz |  |
|  | ±40/200MHz | ≦-40dBc/30KHz |  |
| EVM |  | ≦ 8 % | ≦ 8% |
| Frequency Stability |  | ≦ ±0.01 ppm |  |
| Noise Figure |  | ≦ 8 dB |  |
| VSWR |  | ≦ 1.8 |  |
| System Delay |  | ≦ 4.5 µs |  |
| RF Connector |  | N-Female |  |
| Impedance |  | 50 Ω |  |
| Power Supply |  | AC 100/ 240 V |  |
| Power Consumption |  | ≦ 75 W |  |
| Dimensions |  | 234*135*52mm |  |
| Weight |  | ≦ 3 kgs |  |
| IP Rating |  | IP40 |  |
| Operating Temperature |  | -25 °C to 55 °C |  |
| Control & Monitoring | Local | Type-C port/WIFI(Optional) |  |

Technical specification is subject to change without prior notice.