5G-4G Triple Band-adjustable Digital Pico Repeater

Model: DS-DWN3500-20

## Product Features

- NSA/SA Support: Compatible with both Non-Standalone and Standalone 5G networks.

- Accurate Synchronization: Built-in 5G base band synchronous detection module (SDM) ensures precise timing with the donor base station.

- Multi-band support offers flexibility to meet various network demands.

- Fine-grained control is achieved through individual sub-band activation/deactivation.

- Intelligent algorithms effectively mitigate uplink interference.

- Independent gain control further optimizes performance.

- Self-control mechanisms minimize interference during new base station deployments.

- NMS facilitates remote/local control and monitoring.

## Application Scenario

## Block Diagram

## Technical Specifications

| P/N:DS-DWN3500-20 |  | B3 (1800) | B1 (2100) | N78 (3500) |
| --- | --- | --- | --- | --- |
| Frequency Range | Uplink (adjustable) | 1735.1 – 1754.5 MHz | 1920-1940 MHz | 3600-3700 MHz |
|  | Downlink (adjustable) | 1830.1 – 1849.5 MHz | 2110-2130 MHz | 3600-3700 MHz |
| Bandwidth (adjustable) |  | 19.4 MHz | 20 MHz | 100 MHz |
| Number of Sub-bands |  | 1 | 1 | 1 |
| Sub -Band Adjustment Range |  | 0.2-19.4 MHz | 0.2-20 MHz | 5-100 MHz |
| Max. Gain | Uplink | 70±3 dB | 70±3 dB | 70±3 dB |
|  | Downlink | 75±3 dB | 75±3 dB | 75±3 dB |
| Automatic Gain / Level Control (ALC) |  | ≥ 42dB |  |  |
| Gain Flatness (per sub-band) |  | ≤ ±3.5 dB (peak-to-peak) |  |  |
| Max. Input Power Without Damage |  | 0 dBm |  |  |
| Output Power | Uplink | 20±2 dBm | 20±2 dBm | 20±2 dBm |
|  | Downlink | 20±2 dBm | 20±2 dBm | 20±2 dBm |
| Out of Band Gain |  | 3GPP compliant |  |  |
| Spurious Emission |  | 3GPP compliant |  |  |
| ACLR |  | 3GPP compliant |  |  |
| EVM |  | ≤ 8% | ≤ 8% | ≤ 3.5% |
| Frequency Stability |  | ≤ ±0.01 ppm |  |  |
| Noise Figure |  | ≤ 8 |  |  |
| VSWR |  | ≤ 2 |  |  |
| System Delay |  | ≤ 5 µs |  |  |
| RF Connector |  | N-Female |  |  |
| Impedance |  | 50 Ω |  |  |
| Power Supply | Input | AC 90~264 V, 50/ 60 Hz |  |  |
|  | Output | DC 5 V/ 7 A |  |  |
| Power Consumption |  | ≤ 30 W |  |  |
| Dimensions |  | 234*240*52 mm |  |  |
| Weight |  | ≤ 3 kgs |  |  |
| IP Rating |  | IP35 |  |  |
| Operating Temperature |  | -10 °C to 50 °C |  |  |
| NMS | Local | Via RJ45 |  |  |
|  | Remote | Via Cloud-based NMS (optional) |  |  |