500mW 700+1800+2100MHz Repeater

Model:  AT012-L7DW-27

## Product Features:

- Improve any three mobile networks at the same time

- LCD display the input/output signal strength

- Auto isolation detect function

- Auto gain control ,auto level control

- Auto uplink noise reduction,avoid disturbing to BTS

- Smart LCD to guide the installation

- Metal industrial design

## Working Schematic Diagram

Application Scenario:

## Technical Specifications:

| P/N: AT012-L7DW-27 |  | B3 (1800MHz) | B1 (2100MHz) | B28(700MHz) |
| --- | --- | --- | --- | --- |
| Frequency Range | Uplink | 1710-1785MHz | 1920-1980MHz | 703-748MHz |
|  | Downlink | 1805-1880MHz | 2110-2170MHz | 758-803MHz |
| Bandwidth |  | 75MHz | 75MHz | 35MHz |
| Max. Gain | Uplink | 75±3 dB | 75±3 dB | 75±3 dB |
|  | Downlink | 85±3 dB | 85±3 dB | 85±3 dB |
| Manual Gain Control |  | 31 dB in step of 1 dB |  |  |
| Automatic Gain Control |  | ≧ 20 dB |  |  |
| Gain Flatness (per sub-band) |  | ≦ ±3 dB (peak-to-peak) |  | ≦ ±3 dB (peak-to-peak) |
| Max. Input Power Without Damage |  | 0 dBm |  |  |
| Output Power | Uplink | 20±2 dBm | 20±2 dBm | 20±2 dBm |
|  | Downlink | 27±2 dBm | 27±2 dBm | 27±2 dBm |
| Spurious Emission | 9KHz-1GHz | ≦ -36dBm |  |  |
|  | 1GHz-12.75GHz | ≦ -30dBm |  |  |
| ACRR | ±20/100MHz | ≦-36dBc/30KHz |  | ≦-36dBc/30KHz |
|  | ±40/200MHz | ≦-40dBc/30KHz |  | ≦-40dBc/30KHz |
| EVM |  | ≦ 8%， |  |  |
| Frequency Stability |  | ≦ ±0.01 ppm |  |  |
| Noise Figure |  | ≦ 8 dB |  |  |
| VSWR |  | ≦ 2.5 |  |  |
| System Delay |  | ≦ 8µs |  |  |
| RF Connector |  | N-Female |  |  |
| Impedance |  | 50 Ω |  |  |
| Power Supply |  | AC 90-240 V, 50/ 60 Hz |  |  |
| Power Consumption |  | ≦ 200W |  |  |
| Dimensions |  | 350*268*58mm |  |  |
| Weight |  | ≦ 12kg |  |  |
| IP Rating |  | IP40 |  |  |
| Operating Temperature |  | -25 °C to 55 °C |  |  |
| Control & Monitoring | Local control | WIFI/Via RJ45(Optional) |  |  |
|  | Remote control | LTE modem(Optional) |  |  |

Technical specification is subject to change without prior notice.