Tetra 412MHz Repeater System

Model: RF37-UHF412B

## Product Features:

- LCD display the input/output signal strength.

- Auto isolation detection.

- Improve any three mobile networks at the same time.

- Built-in Auto Gain Control (AGC)& Auto Level Control (ALC).

- Center frequency movable.

- NMS Lite for remote/local control and monitoring(Optional)

## Working Schematic Diagram

Application Scenario:

## Technical Specifications:

| Items |  | Specifications |
| --- | --- | --- |
| System |  | VHF |
| Frequency Range | Uplink(Rx) | 412-417MHz |
|  | Downlink(Tx) | 422-427MHz |
| Bandwidth |  | 5MHz |
| Maximum Output Power | Uplink | 25±2dBm |
|  | Downlink | 37±2dBm |
| Maximum Gain |  | 90±3dB |
| AGC Range |  | ≥20dB |
| Maximum Input power(Non-Destructive) |  | ≤-10dBm |
| VSWR |  | ≤ 2.4 |
| System Delay |  | ≤ 6 μs |
| I/O Impedance |  | 50 Ω |
| Noise Figure |  | ≤ 8dB |
| Spurious Emission |  | 9kHz~ 1GHz: ≤ -36dBm |
|  |  | 1GHz~ 12.75GHz:≤-30dBm |
| Third-order Inter-Modulation |  | ≤ -40dBm |
| RF Connector |  | 2*N-Female |
| Power Consumption |  | ≤ 350W |
| Power Supply |  | Input:AC100~ 240V, 50/60Hz |
| Dimensions |  | 512*461*232mm |
| Weight |  | ≤ 25kg |
| Alarm Monitoring System |  | Alarm for Uplink Self-Oscillation |
| Isolation Detection |  | Isolation Check During Boot Time |
| Operating Temperature |  | -25 ~ +50 °C |
| Application |  | Indoor or Outdoor(IP65) |
| Relative Humidity Range |  | ≤95%(Non condensing) |
| Mounting |  | Wall Mounting |